GLUE HYBRID TO PEDESTAL

Stage 1

Identify hybrid and orientation, check DB for Authorisation

Visual inspection of glue area on hybrid

Make sure hybrid is the correct way round in the handling frame

Identify Pedestal and clean

Stage 2

Accessory list: - 	10cc syringe (check for cracks)

Green needle

Black plastic cap

Araldite 2011 (two pack)

Stirrer

Cut down beaker

Hybrid 32 (scrap test piece)

Clean room wipes

Isopropyl

Kapton tape

Ø220µm wire

Tools			Hybrid vacuum chuck

Hybrid vacuum chuck spacer

Pedestal mount bracket

Pedestal glue dispense plate

Torque driver set to 120cnm

Small screw driver

Feeler gauge 406µm

Small tool makers clamp

2µm clock

Clippers

Wire spacers

Check tooling & clean

Check vacuum paper covers recess

Check vacuum is plugged in, switch on and working

Stage 3

Dry run of hybrid on vacuum chuck to check fit and that vacuum “grabs” hybrid

Quick alignment of sensor in frame

Stage 4

Set glue station: -	Teaching pendent: -	Program 15 (3x PEDESTAL LINES)

Teaching mode on

Dispenser: -		Program 1

Barrel 10cc

Pull back 1000 (or 1500)

Dwell  250 (or 200)

Dispense 0.038cc

Jog 2000

Remote off

Mix 10g of araldite and load in syringe and clean

Load syringe on to dispenser

Jog to engage head and remove air bubbles

Place Glueplate on stand

Load hybrid 32 on to Glueplate

Set needle height and check over dispensed length

Set Dispenser to “Remote on” and Teaching pendent to “Run mode”

Run station until happy with glue beads dispensed on to test Hybrid 32.

Stage 5

Load Hybrid in frame on to Glueplate side to be glued up

TEACHING PENDENT IS SET TO PROG. 15 (3x PEDESTAL LINES)

Wipe needle and push GREEN button

Check glue dispensed is OK

Stage 6

Load Hybrid in frame on to vacuum chuck and vac down

Release Frame

Snip off tags (second person must apply light pressure to hybrid between connectors

Tape wires in to place

Mount pedestal and attach clock (check movement of clock with feeler gauge)

Lower pedestal to just above glue

Alignment of Hybrid

Lower pedestal until clock moves 15 µm

Final alignment of Hybrid