Transport of End-cap C

Peter Cooke, Richard French, Stephen Haywood, Neil Jackson, Peter Sutcliffe

Tech Spec   	ATL-IS-CS-0016

Procedures 	ATL-IS-CS-0023

Guiding principle:

Copy the procedures used by Barrel: CERN/Danzas

See:   http://hepwww.rl.ac.uk/atlas-sct/engineering/ec/transport/

Procedures – see ATL-IS-CS-0023

Preliminary Test Run

See talk on Contractual Status

Before

- Organise Lorry – needs at least 2 weeks

- Just before, check weather

- When Lorry arrives, undertake “Final Test Run” with empty Lorry and Shock-log (if desired)

Preparation of EC – see Peter Cooke for more details

- Move electronics & CMM out of way

- Build Transport Frame around Assembly Frame

- Attach Inner Box to Assembly Frame

- Cover Services & Thermal Plug with ESD-safe film

- Ensure suitable electrical connections within Frame

- Attach inner Shock-log

- Move over Pallet

- Swap wheels from Transport Frame to Pallet

- Attach Outer Box to Pallet

- Attach outer Shock-log

Movement out of Clean-room – from Peter, may need updating

- Add cling-film if concerned about rain/snow.

Loading of End-cap

Responsibility of Andy Park

- EC should be placed to one side and wheeled into centre

- Wheels should be raised

- EC should be secured with load-lock bars

- Driver should be given documents and pep talk

Travel to CERN

- After ~30 mins, Lorry will stop and the Shock-logs checked

- Lorry will go via Channel Tunnel

- Lorry will enter France on weekday and will proceed to Bat 904 in Prevessin for customs clearance

- Finally, the lorry will proceed to SR1

Unloading at CERN

- EC will be placed at entrance of SR1

- Remove Outer Box

- Examine Shock-logs immediately

- Swap wheels from Pallet to Transport Frame

- Wheel Box off Pallet and into Clean Room

- Examine End-cap

Transportation Conditions

New Lorry:

Request:

- Acceleration – < 3g

- Orientation – < 10o

- Temp – 20  3oC

- Humidity – < 80% RH, prefer ~40%